NATIONAL NUCLEAR DATA CENTER

Bldg. 197D

Brookhaven National Laboratory

P. O. Box 5000

Upton, NY  11973-5000  U.S.A.

(Internet) "NNDC@BNL.GOV	Telephone: (516)344-2902

FAX: (516)344-2806

Memo CP-C/291

DATE:	July 24, 2001

TO:	Distribution

FROM:	V. McLane

SUBJECT:	Differential neutron multiplicity

I have received two data sets that measure neutron multiplicity as a function of angle and secondary energy for which I need to introduce a new quantity and units.

Proposed dictionary additions follow.

Dictionary 25 (UNITS)

P/MEVMUCSR	Particles/(MeV muC sr)

Dictionary 36 (Quantities)

,MLT/DA/DE,,TT	Diff. particle multiplicity d/dA/dE

I have attached a sample coded entry.  Has anybody else encountered similar data?

Distribution:

M. Chiba, Sapporo

F. E. Chukreev, CAJaD

S. Dunaeva, Sarov

O. Gritzay, KINR

K. Kato, JCPDG

M. Kellett, NEADB

V. N. Manokhin, CJD

S. Maev, CJD

O. Schwerer, NDS

S. Takács, ATOMKI

F. T. Tárkányi, ATOMKI

V. Varlamov, CDFE

Zhuang Youxiang, CNDC

NNDC File

TRANS

ENTRY            C0825

SUBENT        C0825001

BIB

INSTITUTE  (1USAOHO)

REFERENCE  (J,NSE,129,175,1998)

AUTHOR     (T.N.Massey,S.Al-Quraishi,C.E.Brent,J.F.Guillemette,

S.M.Grimes,D.Jacobs,J.E.O'Donnell,J.Oldendick,

R.Wheeler)

TITLE      A Measurement of the 27Al(d,n) Spectrum for Use in

Neutron Detector Calibration

FACILITY   (ACCEL,1USAOHO) Ohio tandem accelerator.

SAMPLE     Thick aluminum foils.

METHOD     (TOF)

DETECTOR   (FISCH) 235U thick and thin foils.

ANALYSIS   Results of measurements on 2 thick foils combined.

MONITOR    (92-U-235(N,F),,SIG) Data taken from ENDF/B-VI.

CORRECTION Corrected for background, and absorption on neutrons

by aluminum of chamber, cadmium shield, and air.

ERR-ANALYS (ERR-S) Statistical uncertainty.

Normalization uncertainty of 4.9% is the dominant

source of uncertainty.

STATUS     Data received by email from T. Massey, 17 August 1999.

HISTORY    (20010731C) VM

ENDBIB

NOCOMMON

ENDSUBENT

SUBENT        C0825002

BIB

REACTION   (13-AL-27(D,X)0-NN-1,,MLT/DA/DE,,TT)

ENDBIB

COMMON

EN         ANG

MEV        ADEG

7.44     120.

ENDCOMMON

DATA

E          DATA       ERR-S

MEV        P/MEVMUCSR P/MEVMUCSR

0.2200   3.5395E+07 6.0147E+06

0.2700   4.1417E+07 5.4801E+06

0.3200   5.6495E+07 5.1177E+06

…

13.7094   1.9483E+05 5.8838E+04

ENDDATA

ENDSUBENT

ENDENTRY