Nuclear Data Section

International Atomic Energy Agency

P.O.Box 100, A-1400 Vienna, Austria

Memo CP-D/522

Date:		18 September 2008

To:		Distribution

From:		N.Otsuka, S.Dunaeva

Subject:		Usage of data headings and explanation under ERR-ANALYS

- Usage of data headings for uncertainty independent variable

The current coding rules for major data headings for uncertainties are summarized below (See LEXFOR “Errors” for more details):

| Heading | Usage |
| --- | --- |
| ERR-T | Total uncertainty which components are also given under ERR-S, ERR-SYS, ERR-n, MONIT-ERR etc. |
| ERR-S | Statistical uncertainty |
| ERR-SYS | Total systematic uncertainty (partial systematic uncertainties are known or unknown) |
| ERR-1, ERR-2, … | Partial systematic uncertainty except uncertainty in monitor reaction cross section. |
| MONIT-ERR | Uncertainty in monitor reaction cross section |
| DATA-ERR | - Uncertainty which property (statistical or systematic) is uncertain for the compiler - Total uncertainty which components are not given under ERR-S, ERR-SYS, ERR-n, MONIT-ERR etc. |
| DATA-ERR1, DATA-ERR2, … | Similar to DATA-ERR, but more than two components of uncertainties are given by authors. |
| ERR-DIG EN-DIG … | Uncertainty due to digitization. |

If the uncertainty depends on independent variables, and the minimum and/or maximum of the uncertainty are given, they may not be coded under data headings in the COMMON section. They should be explained under ERR-ANALYS in free text.

2. Definition of uncertainty under ERR-ANALYS

The EXFOR format manual “ERR-ANALYS” explains that

“Presence is obligatory, except when not relevant. May contain free text or coded information with free text. However, coded information is obligatory when more than one error field is given in the data set.”

In real compilation, however, we often omit this coded information for uncertainties in independent variable. We would propose that the coded information is optional for uncertainties in independent variables.

“Presence is obligatory, except when not relevant. May contain free text or coded information with free text. However, coded information is obligatory when more than one error field is given in the data set except uncertainties in independent variables.”

Example 1 (some partial errors are known)

…

ERR-ANALYS (ERR-T) Total uncertainty                              X000100100019

- Detector efficiency         (0.5%-1.5%)      X000100100020

- Statistical uncertainty         (<3.0%)      X000100100021

(ERR-1) - Standard cross section           (6.0%)      X000100100022

(ERR-2) – Irradiation geometry             (2.5%)      X000100100023

...

COMMON               2          3                                 X000100100036

ERR-1      ERR-2                                                  X000100100037

PER-CENT   PER-CENT                                               X000100100038

6.0        2.5                                                    X000100100039

ENDCOMMON            3          0                                 X000100100040

DATA                 3          6                                 X000100100009

EN         DATA       ERR-T                                       X000100100010

MEV        MB         MB                                          X000100100011

13.50      1951.      85.                                         X000100100012

13.84      1907.      90.                                         X000100100013

...

Example 2 (partial errors are unknown)

…

ERR-ANALYS (DATA-ERR) Total uncertainty (no detail is given)      X000200100019

…

NOCOMMON             0          0                                 X000200100036

ENDCOMMON            3          0                                 X000200100040

DATA                 3          6                                 X000200100009

EN         DATA       DATA-ERR                                    X000200100010

MEV        MB         MB                                          X000200100011

13.50      1951.      85.                                         X000200100012

13.84      1907.      90.                                         X000200100013

…

Example 3 (Digitized data points without error bars)

…

ERR-ANALYS Absolute uncertainty is less than 30%.                 X000300100019

STATUS     (CURVE) Digitized from Fig.3

COMMON               1          3                                 X000100100036

ERR-DIG                                                           X000100100037

PER-CENT                                                          X000100100038

6.0                                                               X000100100039

ENDCOMMON            3          0                                 X000100100040

DATA                 3          6                                 X000300100009

EN         DATA                                                   X000300100010

MEV        MB                                                     X000300100011

13.50      1951.                                                  X000300100012

13.84      1907.                                                  X000300100013

…

Distribution:

a.mengoni@iaea.org

a.nichols@iaea.org

blokhin@ippe.ru

chiba@earth.sgu.ac.jp

claes.nordborg@oecd.org

ganesan@barc.gov.in

gezg@ciae.ac.cn

hasegawa@nea.fr

henriksson@near.fr

hongwei@ciae.ac.cn

jhchang@kaeri.re.kr

kaltchenko@kinr.kiev.ua

katakura.junichi@jaea.go.jp

kato@nucl.sci.hokudai.ac.jp

kirarlyb@atomki.hu

l.vrapcenjak@iaea.org

manokhin@ippe.ru

mmarina@ippe.ru

mwherman@bnl.gov

nklimova@kinr.kiev.ua

n.otsuka@iaea.org

nrdc@jcprg.org

oblozinsky@bnl.gov

ogritzay@kinr.kiev.ua

otto.schwerer@aon.at

samaev@obninsk.ru

s.babykina@polyn.kiae.su

s.dunaeva@iaea.org

stakacs@atomki.hu

stanislav.hlavac@savba.sk

taova@exped.vniief.ru

tarkanyi@atomki.hu

varlamov@depni.sinp.msu.ru

vlasov@kinr.kiev.ua

vmclane@optonline.net

v.zerkin@iaea.org

yolee@kaeri.re.kr