- Japan Charged-Particle Nuclear Reaction Data Group

- Division of Physics, Graduate School of Science

- Hokkaido University

- 060-0810 Sapporo, JAPAN

E-mail: nrdf@jcprg.org                                                                                          Telephone    +81(JPN)-11-706-2684

Internet: http://www.jcprg.org/                                                                               Facsimile    +81(JPN)-11-706-4850

- Memo CP-E/040

| Date: | April 26, 2004 |
| --- | --- |
| To: | Distribution |
| From: | OTSUKA Naohiko and KATO Kiyoshi |
| Subject: | Thick target product yields (E1756, E1759, E1788) |
| Reference: | CP-C/334, CP-D/391 |

- Along the recent discussion of thick target yields, we made some corrections to E1756 and E1759, and compiled a new entry E1788. In these entries, we compiled quantities measured for SF1(SF2,X)0-NN-1 as follows:

| - Quantity | - Unit | - Subentries |
| --- | --- | --- |
| - ,PY/DA/DE,,TT | - P/IN/MEVSR | - E1756.002-015 - E1759.002-013 - E1788.002-013 |
| - ,PY/DA,,TT | - PRD/INC/SR (new) | - E1756.016-029 |
| - ,PY/DA,,TT/IPA (new) | - PRD/INC | - E1756.030-043 - E1759.014-017 |

In terms of EXFOR, a neutron in the final state of an inclusive reaction (i.e. SF3=X) is not an outgoing particle (SF3) but a reaction product (SF4). Therefore we did two corrections:

- Parameter code is PY rather than MLT.

- Unit code is PRD rather than PRT.

- ”P” in “P/IN/MEV/SR” is kept, but read as “product” .

- Namely, we stick “SF3 vs. SF4 rule” even for the light particle emission in an inclusive measurement. We propose one unit code and one quantity code:

- Dictionary 25 (Units)

- PRD/INC/SR     1/A  products/inc. projectile/ steradian

- Dictionary 36 (Quantities)

,PY/DA,,TT/IPA YLD  Diff. product d/DA for thick target integrated over angular range

Summary for codes for thick target product yields and thick target yield multiplicity:

| Outgoing particle (SF3) |  | Reaction product (SF4) |  | Dimension |
| --- | --- | --- | --- | --- |
| ,MLT,,TT | PRT/INC | ,PY,,TT | PRD/INC | YLD |
| ,MLT/DA,,TT | PRT/INC/SR | ,PY/DA,,TT | (PRD/INC/SR) | 1/A |
| ,MLT/DE,,TT | PRT/IN/MEV | (not used yet) | (not used yet) | 1/E(*) |
| ,MLT/DA/DE,,TT | P/IN/MEVSR | ,PY/DA/DE,,TT | P/IN/MEV/SR | 1/AE |
| ,TTY/MLT | PART/MUAHR | ,TTY/PY | PRD/MUAHR PRD/MUCOUL | PYT(†) |
| (,TTY/MLT/DA) | (PRT/MUC/SR) | (,TTY/PY/DA) | (PRD/MUC/SR) | PYTA |
| ,TTY/MLT/DA/DE | P/MEVMUCSR | (,TTY/PY/DA/DE) | P/MEVMUCSR | YAE(‡) |

Note: Codes with brackets are not in present dictionary but proposed in recent memos.

- ,TTY/MLT/DA   (PYTA) CP-D/389  (with PAR in SF5)

-   PRT/MUC/SR   (PYTA) CP-D/389

- ,PRD/INC/SR   (1/A)   CP-E/040

- ,TTY/PY/DA    (PYTA) CP-D/391  (with PAR in SF5)

-   PRD/MUC/SR   (PYTA) CP-D/391

- ,TTY/PY/DA/DE (YAE)   CP-C/340

Corrections of the dictionary are needed for the following

*)  YLD is assigned for ,MLT/DE,,TT in the dictionary, must be 1/E.

c.f. F0341.002,004,006,008,010,012,014

†)  TTY is assigned for PART/MUAHR and PRD/MUAHR in the dictionary, must be PYT.

c.f. B0173.002, C0006.003

‡) TD2 is assigned for ,TTY/MLT/DA/DE and P/MEVMUCSR in the dictionary, must be YAE.

Distribution:

| S. Babykina, CAJaD | J.H. Chang, KAERI | M. Chiba, JCPRG | F.E. Chukreev, CAJaD |
| --- | --- | --- | --- |
| S. Dunaeva, NDS | Z.G. Ge, CNDC | O. Gritzay, KINR | A. Hasegawa, JAERI |
| A. Kaltchenko, KINR | K. Katō, JCPRG | M. Kellet, NEA-DB | M. Lammer, NDS |
| M. Lammer, NDS | S. Maev, CJD | V.N. Manokhin, CJD | V. McLane, NNDC |
| M.Mikhaylyukova, CJD | C. Nordborg, NEA-DB | P. Oblozinsky, NNDC | N. Otsuka, JCPRG |
| O. Schwerer, NDS | S. Takacs, ATOMKI | S. Taova, VNIIEF | T. Tárkányi, ATOMKI1 |
| V. Pronyaev, NDS | V. Varlamov, CDFE | M. Vlasov, KINR | M. Wirtz, NDS |
| H.W. Yu, CNDC | V. Zerkin, NDS | Y.X. Zhuang, CNDC | EXFOR, NEA-DB |