- Japan Charged-Particle Nuclear Reaction Data Group

- Department of Physics, Faculty of Science

- Hokkaido University

- 060-0810 Sapporo, JAPAN

E-mail: services@jcprg.org

Internet: http://www.jcprg.org/

Telephone    +81(JPN)-11-706-2684

Facsimile    +81(JPN)-11-706-4850

- Memo CP-E/120 (Revised)

| Date: | October 4, 2007 |
| --- | --- |
| To: | Distribution |
| From: | OTSUKA Naohiko and Hans Henriksson |
| Subject: | Plotting flags in Archive Dictionary 24 (Data headings) |

Columns 50 to 56 of Archive dictionary 24 are used for plotting flags. These flags inform us the role of headings in plotting (independent variable or dependent variable? data point or error bar? etc.). The current rule of these 7 columns however does not cover minimum, maximum, approximate, uncertainty and resolution of one of multiple variables (E1, ANG1, etc). We would like to propose a revision of the rule.

Our current rule for these 7 columns is as follows (See more detail in the dictionary manual prepared by Schwerer in August, 2007.):

Distribution:

| S. Babykina, CAJaD | A. Blokhin, CJD | J.H. Chang, KAERI | M. Chiba, JCPRG |
| --- | --- | --- | --- |
| S. Dunaeva, NDS | S. Ganesan, BARC | Z.G. Ge, CNDC | O. Gritzay, KINR |
| A. Hasegawa, NEA-DB | H. Henriksson, NEA-DB | M. Herman, NNDC | A. Kaltchenko, KINRｑ |
| J. Katakura, JAEA | K. Katō, JCPRG | Y.O. Lee, KAERI | S. Maev, CJD |
| V.N. Manokhin, CJD | V. McLane, NNDC | A. Mengoni NDS | M. Mikhaylyukova, CJD |
| A. Nichols, NDS | C. Nordborg, NEA-DB | P. Obložinský, NNDC | Y. Ohbayasi, JCPRG |
| A. Ohnishi, JCPRG | N. Otuka, JCPRG | V. Pronyaev, CJD | D. Rochman, NNDC |
| O. Schwerer, NDS | S. Tákacs, ATOMKI | S. Taova, VNIIEF | T. Tárkányi, ATOMKI |
| V. Varlamov, CDFE | M. Vlasov, KINR | M. Wirtz, NDS | H.W. Yu, CNDC |
| V. Zerkin, NDS | Y.X. Zhuang, CNDC | EXFOR, NEA-DB |  |

| Column number |  |  |  |
| --- | --- | --- | --- |
| Independent variable | Dependent variable |  |  |
| 50 | 53 | 1:value 2:minimum 3:maximum 4:approximate 5:one of multiple variable | 9: uncertainty or resolution |
| 51 | 54 | 1:numerator 2:denominator | 1:+error 2:+resolution 3:+half resolution 4:+statistical error 5:+partial error |
| 52 | 55 |  | 1:-error 2:-resolution 3:-half resolution 4:-statistical error 5:-partial error |
| 56 |  | 1:CM |  |

However, it is impossible to express minimum, maximum, approximate, uncertainty and resolution of one of multiple variables (E1, ANG1, etc) in the current rule.

Example:

| Column # | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E | 1 |  |  |  |  |  |  |
| E-MAX | 3 |  |  |  |  |  |  |
| E-CM | 1 |  |  |  |  |  | 1 |
| E-ERR | 9 | 1 | 1 |  |  |  |  |
| +E-ERR | 9 | 1 |  |  |  |  |  |
| -E-ERR | 9 |  | 1 |  |  |  |  |
| E-RSL | 9 | 2 | 2 |  |  |  |  |
| DATA |  |  |  | 1 |  |  |  |
| DATA-ERR |  |  |  | 9 | 1 | 1 |  |
| ERR-S |  |  |  | 9 | 4 | 4 |  |
| ERR-1 |  |  |  | 9 | 5 | 5 |  |
| E1 | 5 |  |  |  |  |  |  |
| E1-MAX | ??? |  |  |  |  |  |  |
| E1-ERR | ??? |  |  |  |  |  |  |

The last two examples show that we cannot express uncertainty or resolution of “one of multiple variable” cannot express in the current rule.

Proposal

Option 1: Delete “one of multiple variables”

E1, E2, ANG1, ANG2,… is “one of multiple variable” in the current rule. However E, ANG,… can also be “one of multiple variable” (Example: E and ANG in double differential cross section). Therefore deletion of “one of multiple variables” is one option.

| Column number |  |  |  |
| --- | --- | --- | --- |
| Independent variable | Dependent variable |  |  |
| 50 | 53 | 1:value 2:minimum 3:maximum 4:approximate 5:one of multiple variables | 9:uncertainty or resolution |
| 51 | 54 | 1:numerator 2:denominator | 1:+error 2:+resolution 3:+half resolution 4:+statistical error 5:+partial error |
| 52 | 55 |  | 1:-error 2:-resolution 3:-half resolution 4:-statistical error 5:-partial error |
| 56 |  | 1:CM |  |

Below is an example of plotting flags according the first option:

| Column # | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E | 1 |  |  |  |  |  |  |
| E-MAX | 3 |  |  |  |  |  |  |
| E-CM | 1 |  |  |  |  |  | 1 |
| E-ERR | 9 | 1 | 1 |  |  |  |  |
| +E-ERR | 9 | 1 |  |  |  |  |  |
| -E-ERR | 9 |  | 1 |  |  |  |  |
| E-RSL | 9 | 2 | 2 |  |  |  |  |
| DATA |  |  |  | 1 |  |  |  |
| DATA-ERR |  |  |  | 9 | 1 | 1 |  |
| ERR-S |  |  |  | 9 | 4 | 4 |  |
| ERR-1 |  |  |  | 9 | 5 | 5 |  |
| E1 | 1 |  |  |  |  |  |  |
| E1-MAX | 3 |  |  |  |  |  |  |
| E1-ERR | 9 | 1 | 1 |  |  |  |  |

Option 2: Keep “one of multiple variables” category

We never met “minimum error”, “maximum error” and “approximate error” etc. (e.g. ANG-ERR-MAX, ANG-RSL-MIN, …). Therefore the following scheme covers all headings in dictionary 24 (One exception: E-NM-ERR cannot be expressed by this second option.).

| Column number |  |  |  |
| --- | --- | --- | --- |
| Independent variable | Dependent variable |  |  |
| 50 | 53 | 1:value 2:one of multiple variables | 8:uncertainty or resolution for value 9:uncertainty or resolution for one of multiple variables |
| 51 | 54 | 1:numerator 2:denominator | 1:+error 2:+resolution 3:+half resolution 4:+statistical error 5:+partial error |
| 52 | 55 | 1:minimum 2:maximum 3:approximate | 1:-error 2:-resolution 3:-half resolution 4:-statistical error 5:-partial error |
| 56 |  | 1:CM |  |

Below is an example of plotting flags according the second option:

| Column # | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E | 1 |  |  |  |  |  |  |
| E-MAX | 1 |  | 2 |  |  |  |  |
| E-CM | 1 |  |  |  |  |  | 1 |
| E-ERR | 9 | 1 | 1 |  |  |  |  |
| +E-ERR | 9 | 1 |  |  |  |  |  |
| -E-ERR | 9 |  | 1 |  |  |  |  |
| E-RSL | 9 | 2 | 2 |  |  |  |  |
| DATA |  |  |  | 1 |  |  |  |
| DATA-ERR |  |  |  | 9 | 1 | 1 |  |
| ERR-S |  |  |  | 9 | 4 | 4 |  |
| ERR-1 |  |  |  | 9 | 5 | 5 |  |
| E1 | 2 |  |  |  |  |  |  |
| E1-MAX | 2 |  | 2 |  |  |  |  |
| E1-ERR | 8 | 1 | 1 |  |  |  |  |